gbsv Mini-Challenge 1

2026-02-24

General Information

In this mini-challenge, a profile of a country (Steckbrief) will be compiled in the form of experiments in foundations in image and signal processing to assess aspects of LO1-LO3. Each student will submit a unique solution with individual data and analysis. This means you will choose images and signals themed suitably for your selected country and the tasks assigned to you. Unless otherwise mentioned, existing libraries may be used.

Note: Certain tasks are kept open-ended. Set your own frame for the limitation or ask if the boundaries are not clear to you. As a rule of thumb one day should take you around 2h +/-30min per day. Appropriate data selection, pinpoint creativity, moderate diversity, and critical thinking are sought rather than extensive submissions.

Version Individual Rhythm

- Conduct your MC according to your own schedule/rhythm

- Mandatory check-in (review meeting): MC is only graded if a check-in was conducted.

Version 15-Day Challenge

- Profit from regular feedback

- Earn 1 bonus credit per submitted day if you submit at least 10 days

- Bonus credits: up to +0.33 added to final grade (scaled to 15; capped at 6.0 for maximum grade)

- Revisions are allowed until day 15 if changes are clearly highlighted in color. Code does not need to be colored, but a colored text tag should mark start and end of code changes.

- Submission deadline: all entries must be submitted by day 15 (per challenge)

Submission and Grading

- Submission are conducted via Microsoft Teams Assignments in the gbsv Teams.

- Deliverables:

  - Submit HTML export of your submission (one notebook per section 1.1-1-3)

  - Link to code and results incl. signals/images, if applicable

  - No zip files are accepted!

- Make sure that audios are playable within notebook exports

- See grading rubrics, gbsv interpretations and indicators for strong vs. weak performance

- Submissions received after the deadline or submitted via email will not be considered.

- Assessment is based on the quality of your technical framing and justification, not on whether you adopt a provided example use case or develop your own.

  - Sampling Theorem

Learning Objectives

- Conceptual: Apply the Nyquist-Shannon theorem to explain aliasing in a domain-specific context.

- Methodological: Identify a domain-relevant signal where sampling affects information fidelity and derive its required sampling rate from the bandwidth.

- Technical: Implement and compare sampling above, at, and below the Nyquist rate.

- Analytical: Quantitatively assess sampling distortion using appropriate metrics.

- Reflective: Evaluate practical risks and real-world implications of incorrect sampling decisions.

Day 1 – Data & Domain

On this day, you will define the conceptual and application framework of this MC task. First, describe a realistic use case in which sampling plays a critical role and briefly justify why it is relevant to your selected country. Then formulate a precise problem statement that explicitly addresses the sampling rate, identifies what may go wrong if it is chosen inadequately, and clarifies which aspect of signal fidelity is essential in your context. Next, state the experimental objective by defining what you intend to investigate regarding sampling behavior and explain how this investigation helps address the technical vulnerability identified in your problem statement.

Then select and describe a representative one-dimensional signal suitable for your use case, including its physical origin, unit of measurement, acquisition method, and expected frequency characteristics.

Interpret the Nyquist frequency and Nyquist rate for your specific signal and application. Base your explanation on the physical meaning and the observable temporal or spatial variations in your data. Discuss practical consequences of too low and unnecessarily high sampling rates in your use case. No mathematical derivations or implementations are required at this stage.

Start a new notebook for this block.

Reference examples: The following use cases are provided for orientation. You may select one of them and adapt it to your chosen country, or define a suitable alternative within your selected domain.

Ocean Wave Monitoring

Use Case: At a coastal holiday destination, wave height is continuously recorded to assess swimming safety and characterize prevailing sea conditions. The ocean surface motion consists of periodic swell components superimposed with shorter wind-driven waves.

Problem Statement: The wave height signal contains multiple frequency components that describe the energy and dynamics of the sea state. If the sampling rate is chosen too low, faster wave components may be aliased and appear as misleading lower-frequency oscillations, potentially distorting the interpretation of wave conditions. Conversely, excessively high sampling increases storage and transmission demands without adding meaningful information beyond the physical bandwidth of the signal. The critical requirement is the faithful digital representation of the relevant wave frequency range without distortion.

Audio Engineering – Speech Recording

Use Case: Digital speech recordings are used in communication and voice-processing systems. Speech signals contain broadband frequency components that determine intelligibility and phonetic structure.

Problem Statement: Speech signals include frequency components that are essential for preserving intelligibility and phoneme discrimination. If the sampling rate falls below the required threshold, higher-frequency components may be aliased or distorted, altering spectral characteristics and reducing clarity. At the same time, unnecessarily high sampling increases data volume and computational load without improving perceptual quality. The central issue is ensuring accurate spectral representation of the relevant speech bandwidth in the digital domain.

Fitness Tracker During Hiking Holidays

Use Case: During a hiking holiday, a wearable fitness tracker records acceleration signals and optical pulse measurements to monitor movement and cardiovascular activity. These signals contain rhythmic physiological and biomechanical components.

Problem Statement: Acceleration and pulse signals exhibit dominant periodic components, higher-order harmonics, and short-term variations caused by changes in activity. If the sampling rate is insufficient, rapid physiological fluctuations or harmonic components may be aliased or attenuated, leading to inaccurate representation of cadence or heart rate dynamics. Excessively high sampling, however, increases energy consumption and reduces battery life in wearable devices. The key requirement is the reliable digital capture of relevant physiological frequency content within practical device constraints.

Wind Turbine Vibration Monitoring

Use Case: Vibration sensors installed in a wind turbine record mechanical oscillations associated with rotor rotation and structural dynamics. These measurements are used to characterize operational behavior.

Problem Statement: Mechanical vibration signals contain characteristic frequency components related to rotational speed and structural resonance modes. If the sampling rate is too low, higher-frequency vibration components may be aliased and misrepresented, distorting spectral interpretation of turbine dynamics. If the sampling rate is excessively high, data storage and processing requirements increase without capturing additional physically meaningful information. The essential requirement is an accurate digital representation of the vibration bandwidth relevant to mechanical behavior.

Day 2 – Methodological

On this day, you will establish the theoretical and methodological foundation for your sampling analysis. Formally state the Nyquist–Shannon Sampling Theorem and explain its underlying assumptions, particularly the requirement of band limitation.

Based on the signal defined in Day 1, determine its bandwidth and clearly justify how this bandwidth is identified (e.g., analytical derivation, known signal properties, or justified approximation). From this bandwidth, explicitly derive both the Nyquist frequency and the corresponding Nyquist rate. State the minimum required sampling rate mathematically and document each step of your reasoning.

In addition, explain how you will later vary the sampling rate in a structured way to investigate sampling behavior and potential aliasing effects. Justify why these parameter choices are methodologically appropriate for your signal and use case.

The focus of this day is on mathematical rigor and methodological transparency; no simulations, visualizations, or experimental demonstrations should be included.

Day 3 – Technical Implementation

On this day, you will implement the computational framework required to investigate sampling behavior in your defined signal. Based on the methodological decisions from Day 2, generate a high-resolution reference version of your signal that serves as a continuous approximation.

Implement discrete sampling at systematically chosen sampling rates, including at minimum: a rate above the Nyquist rate, exactly at the Nyquist rate, and below the Nyquist rate. Ensure that all parameters are clearly defined and traceable.

Visualize the sampled signals in the time domain and, where appropriate, in the frequency domain. Explicitly demonstrate aliasing effects in the undersampled case. The objective is to produce a technically correct implementation that reflects the methodological framework defined previously.

No quantitative evaluation or extended interpretation is required at this stage; the focus lies on clean implementation, structured experimentation, and traceability.

Day 4 – Evaluation

On this day, you will systematically evaluate the sampling configurations implemented in Day 3. Using 1-2 quantitative evaluation approaches (e.g., numerical metrics and/or quantitatively interpretable visualizations), assess the differences between the high-resolution reference signal and the sampled versions. Suitable approaches may include reconstruction error, frequency deviation, amplitude distortion, or measurable spectral characteristics.

The choice of evaluation approach must be explicitly justified. Explain why the selected metric or extracted visualization quantity is appropriate for your signal characteristics and for the objectives defined in your use case. Clearly specify what aspect of signal quality is captured (e.g., amplitude fidelity, frequency preservation, distortion).

Compare the cases above, at, and below the Nyquist rate in a structured and traceable manner using a clearly defined baseline configuration. Quantify performance differences consistently across all configurations and report values with appropriate units or normalization. Identify the sampling rate at which distortion becomes significant and quantify the magnitude of information loss.

If the signal represents audio data, playback of the sampled signals may be included as a supplementary perceptual evaluation. In this case, briefly describe audible differences (e.g., aliasing artifacts, distortion, loss of clarity) in relation to the quantitative results. Audio playback does not replace the required metric-based or quantitatively measurable evaluation but may support interpretation in use cases where perceptual quality is relevant.

The focus of this day is on objective, quantitative assessment and structured comparison. No new simulations, theoretical derivations, or interpretative discussion should be introduced.

Day 5 – Analytical & Communication

On this day, you will observe, interpret, and discuss your findings in direct relation to your initial problem statement and defined use case. Based on the quantitative evaluation from Day 4, analyze how different sampling rates affect signal fidelity and how these effects translate into domain-specific consequences.

Structure your analysis into observations, interpretation, and discussion. Discuss the practical applicability of your results: Which sampling rate is appropriate and realistic in your chosen context, and why? What risks arise if the sampling rate is chosen incorrectly? Critically assess how the theoretical assumptions of the Nyquist-Shannon theorem align with real-world conditions in your application domain.

Present your analysis in a concise, technically precise, and logically structured form (approximately 180-220 words). All arguments must clearly connect back to the original problem statement. No new simulations or derivations should be introduced.